Meccanoid AI Assistant

Functional Requirements

- The system shall convert user speech to text in real time.

- The system shall send user text to a local LLM for response generation.

- The system shall output verbal replies through text to speech

- The system shall detect a wake-up word (“Hey Mecca”) before processing commands

- The system shall classify user intent from natural language input.

- The system shall issue commands to the Meccanoid G15 robot related to motors.

- The robot shall gesture-wave, nod, turn, walk, or even dance-on demand.

- The system shall synchronize gestures with speech when appropriate.

- The system shall allow the user to change the wake up word in configuration settings.

- The system shall provide the ability to increase, decrease, or mute the robot’s voice output

- The system shall log information about user interactions and commands.

- The system shall support at least five user interaction scenarios, including greeting, movement, Q&A, jokes, and help.

Non-Functional Requirements

- Speech recognition shall be completed within 1 second under normal conditions.

- LLM response generation will be completed within 2 seconds.

- The system shall sustain at least 85% speech recognition accuracy in quiet environments.

- The system shall not require any internet access, having the ability to load and run completely offline.

- The system shall be modularly deployed, separating AI, speech, and motor control.

Hardware & Integration Requirements

- The system shall communicate with the Meccanoid robot using its official or reverse-engineered servo protocol.

- The system shall support USB or Bluetooth-based communication.

- The robot's physical movement shall remain inside safe motor ranges.

- The system shall permit manual motor testing through a developer interface.

- The system shall alert the user when the robot’s battery reaches a low threshold.

- The system shall support changing out AIs or speech libraries with minimal changes to code.

Usability Requirements

- The robot will respond in natural conversational phrasing.

- The system shall not require more than 1 minute to configure its startup.

- Robot states - listening, processing, or error - shall be indicated through LEDs or audio signals.

- The system shall handle misunderstandings using fallback messages (“Can you repeat that?”).

- The system shall allow users to assign a custom name for the robot to introduce itself.

Data and Security Requirements

- All interaction logs shall be stored locally and not in the cloud.

- Sensitive information, such as voice logs or transcripts, shall be deleted upon request.

- If wireless communication is utilized, commands shall be encrypted.

Constraints

- The system shall be implemented with the use of Python for AI integration and C++ for motor control.

- All AI processing must run on local hardware without any external APIs.

- Only free open-source libraries are allowed; the reason is budgetary.

Testing Requirements

- Unit tests for AI, speech-to-text, and motor functions shall be included in the system.

- The robot shall pass 5 predefined interaction test cases, such as greeting, movement, and question/answer.

- The system shall log errors for debugging during testing.

- The robot shall continuously run for at least 15 minutes without any system failure.

Deployment Requirements

39. Installation steps, run instructions, and documentation will go in the GitHub repository.